Landslide Digital Twin Variable And Calibration Matrix

Based on landslide_DT_technical_variables.docx and the active notebook implementation in notebook/Multi_Model_Probability_updated1.ipynb

Purpose. This note restructures the current landslide workflow into a practical technical matrix: raw/observed inputs, derived variables, modelled variables, and a calibration table linking uncertain parameters to observable targets such as SMAP soil moisture, ERA5 SWE, and mapped landslide locations.

Important current notebook state. The active landslide run currently uses local max recharge (`max_recharge`) in the final LandslideProbability call. `routed_recharge_max` is computed but is not the active forcing in that final run cell. The active burn-severity cohesion block also does not currently reduce cohesion values.

# 1. Raw / Observed Inputs

These variables are imported from files or direct external products. They should generally be treated as observations or prepared inputs, not model outputs.

| Variable / file | Type / dims | Units | Source | Role in workflow |
| --- | --- | --- | --- | --- |
| topographic__elevation.asc -> topographic__elevation | [y, x] raster -> [n_nodes] | m | DEM / prepared static layer | Base grid, routing, slope, contributing area |
| soil__thickness.asc -> soil__thickness | [y, x] -> [n_nodes] | m | Prepared soil raster | Controls storage and slope-stability thickness |
| soil__density.asc -> soil__density | [y, x] -> [n_nodes] | kg m^-3 | Prepared soil raster | Used in factor of safety through unit weight |
| soil__internal_friction_angle.asc -> soil__internal_friction_angle | [y, x] -> [n_nodes] | deg | Prepared soil raster | Shear-strength term in infinite-slope model |
| porosity.asc -> porosity | [y, x] -> [n_nodes] | m3 m^-3 | Prepared soil raster | Converts theta to degree of saturation |
| field__capacity.asc -> field__capacity | [y, x] -> [n_nodes] | m3 m^-3 | Prepared soil raster | Converted to field-capacity saturation threshold |
| wilting__point.asc -> wilting__point | [y, x] -> [n_nodes] | m3 m^-3 | Prepared soil raster | Converted to wilting-point saturation threshold |
| soil__saturated_hydraulic_conductivity.asc -> soil__saturated_hydraulic_conductivity | [y, x] -> [n_nodes] | m day^-1 | Prepared soil raster | Hydraulic conductivity used by ecohydrology and landslide component |
| vegetation__plant_functional_type.asc -> vegetation__plant_functional_type | [y, x] -> [n_nodes] | class id | Prepared vegetation raster | Maps to LAI, cover, and vegetation behavior |
| soil__minimum_total_cohesion.asc / mode / maximum | [y, x] -> [n_nodes] | Pa | Prepared strength rasters | Triangular bounds for cohesion uncertainty |
| burn__severity.asc -> burn__severity | [y, x] -> [n_nodes] | class id | External disturbance raster | Scenario conditioning and post-fire interpretation |
| PRISM daily precip ASC stack | [time, y, x] | mm day^-1 | Observed gridded forcing | Daily water input forcing base |
| PRISM daily Tmin / Tmax ASC stack | [time, y, x] | degC | Observed gridded forcing | Snow partition, PET, and ecohydrology forcing |
| SMAPL4_SM_post_fire_mudflow*_daily_pacific.csv | [time] | m3 m^-3 | SMAP L4 product | Initialization and calibration target for AOI-average soil moisture |
| SMAPL4_SM_post_fire_mudflow2_pacific.csv | [timestamp] | m3 m^-3 | SMAP L4 product | Sub-daily comparison against daily means |
| ERA5 SWE series (used in analysis / expected calibration) | [time] | m water eq. or mm | ERA5 | Target for snow-module calibration |
| Mapped landslide locations / initiation polygons | vector geometry | presence / absence | Inventory or lidar interpretation | Target for calibrating landslide probability and post-fire weakening scenarios |

# 2. Derived Variables

These are computed deterministically from raw inputs or intermediate routing logic. They are not directly observed, but they are reproducible given the input stack and notebook choices.

| Derived variable | Computed from | Units / dims | Equation or rule | Why it matters |
| --- | --- | --- | --- | --- |
| drainage_area | DEM + boundary conditions + FlowAccumulator | [n_nodes], m2 | Landlab flow routing | Upslope area and routing diagnostic |
| log_drainage_area | drainage_area | [n_nodes] | log10(max(drainage_area, 1)) | Diagnostic plotting / scaling |
| topographic__slope | DEM | [n_nodes], gradient | topographic__steepest_slope copied to required field name | Required by LandslideProbability |
| topographic__specific_contributing_area | drainage_area, grid.dx | [n_nodes], m | a = A_d / dx | Hydrologic term in relative wetness |
| Aspect, Hillshade | DEM | [n_nodes] | GIS-style terrain derivations | Diagnostics and interpretation only |
| soil__transmissivity | Ksat, soil thickness | [n_nodes], m2 day^-1 | T = Ksat * 2.5 * hs; floor at 0.01 | Static transmissivity proxy and strength/hydrology interpretation |
| vegetation__live_leaf_area_index | PFT lookup | [n_nodes] | grass=1.5, shrub=2.0, tree=4.0, bare=1.0 | Controls PET and vegetation response |
| vegetation__cover_fraction | LAI | [n_nodes] | cover = LAI / 4 | Ecohydrology vegetation cover input |
| soil_moisture__initial_saturation_fraction | SMAP theta0 or midpoint heuristic, porosity | [n_cells] | S0 = theta0 / n or midpoint heuristic | Initial water state for daily run |
| field_capacity_saturation | field__capacity, porosity | [n_cells] | S_fc = clip(theta_fc / n, 0.05, 0.98) | Cell saturation threshold in SoilMoisture |
| wilting_point_saturation | wilting__point, porosity | [n_cells] | S_wp = clip(theta_wp / n, 0.01, 0.95); S_wp <= S_fc - 0.01 | Cell stress threshold in SoilMoisture |
| snow_fraction_arrays | precip, Tmin, Tmax | [time, n_nodes] | linear rain-snow partition between T_snow and T_rain | Partitions storm water into rain vs snow |
| rain_depth_arrays / snow_depth_arrays | precip + snow fraction | [time, n_nodes], mm | P_rain = P(1-fsnow), P_snow = Pfsnow | Liquid and frozen inputs |
| swe_arrays | initial SWE, snow, melt | [time, n_nodes], mm | SWE_t = SWE_{t-1} + snow - melt | Snow storage diagnostic |
| water_input_arrays | rain + melt | [time, n_nodes], mm | W_in = rain + melt | Actual liquid water passed to ecohydrology run |
| mean_recharge / max_recharge | daily recharge arrays | [n_nodes], mm day^-1 | temporal mean / temporal max | Scenario-ready recharge summaries |
| mean_runoff / max_runoff | daily runoff arrays | [n_nodes], mm day^-1 | temporal mean / temporal max | Hydrologic diagnostics |
| routed_recharge_max | max_recharge + flow routing | [n_nodes], recharge-equivalent | surface_water__discharge / drainage_area | Upslope-area-averaged recharge scenario |
| groundwater__recharge_mean / std | local or routed recharge scenario | [n_nodes], mm day^-1 | std = 0.1 * mean in active setup | Direct input to LandslideProbability recharge sampling |

# 3. Modelled Variables And Outputs

These are produced by the ecohydrology and landslide model components during the run.

| Modelled variable | Producer | Units / dims | Physical meaning | Primary use |
| --- | --- | --- | --- | --- |
| soil_moisture__saturation_fraction | SoilMoisture | [time, n_cells], fraction | Degree of saturation in root zone | Hydrologic state and SMAP comparison after multiplying by porosity |
| soil_moisture__root_zone_leakage | SoilMoisture | [time, n_cells], mm | Deep leakage below root zone | Recharge source used for landslide forcing |
| surface__runoff | SoilMoisture | [time, n_cells], mm | Surface runoff generated during daily step | Runoff diagnostics and routing |
| surface__evapotranspiration | SoilMoisture + PET | [time, n_cells], mm | Actual ET | Water-balance diagnostics |
| vegetation__water_stress | SoilMoisture | [time, n_cells], fraction-like | Plant stress indicator | Ecohydrology interpretation |
| surface_water__discharge | FlowAccumulator | [n_nodes], volume / time | Accumulated routed flow | Used to create routed recharge and routed runoff diagnostics |
| soil__mean_relative_wetness | LandslideProbability | [n_nodes], 0-1 | Mean wetness term in infinite-slope stability | Diagnostic landslide-state output |
| soil__probability_of_saturation | LandslideProbability | [n_nodes], 0-1 | Probability that wetness >= 1 in Monte Carlo samples | Secondary saturation-risk output |
| landslide__probability_of_failure | LandslideProbability | [n_nodes], 0-1 | Probability that FS <= 1 | Primary hazard target layer |
| routed_runoff_m3_day | FlowAccumulator post-processing | [n_nodes], m3 day^-1 | Accumulated routed runoff discharge | Hydrologic diagnostic only |

# 4. Calibration Coefficients, Parameters, And Targets

This table separates uncertain coefficients and tunable parameters from fixed raw inputs. The key point is to calibrate parameters against the right observable, not just against the final landslide map.

| Parameter / coefficient | Where it appears now | Affects | Primary calibration target | Secondary target / constraint | Recommendation |
| --- | --- | --- | --- | --- | --- |
| Initial volumetric soil moisture theta0 / initial saturation S0 | Notebook initialization cell | Starting water state for storm run | SMAP daily soil moisture at event start | Early-day modelled soil moisture trajectory | High priority. Calibrate first because it shifts the whole event response. |
| field_capacity_saturation (S_fc) | Derived from field__capacity / porosity | Drainage onset, storage, plant-water response | SMAP daily soil moisture time series | Recharge timing and ET behavior | Treat as soft-calibration target if source products are uncertain. |
| wilting_point_saturation (S_wp) | Derived from wilting__point / porosity | Water-stress threshold and drying behavior | SMAP drying curve shape | Vegetation stress realism | Calibrate jointly with S_fc, not independently in an unconstrained way. |
| ksat_factor = 0.5 | Daily hydro-eco loop | Infiltration, leakage, runoff partition | SMAP soil moisture time series | Recharge magnitude, runoff timing, landslide response pattern | Very high priority. This is an explicit tuning factor in the current notebook. |
| Transmissivity anisotropy factor = 2.5 | T = Ksat * 2.5 * hs | Stability through wetness term R/T | Observed landslide locations / initiation density | Recharge-wetness realism | Important if Ksat and hs are kept fixed. |
| Root-zone depths (zr_grass, zr_shrub, zr_tree, zr_bare) | SoilMoisture component parameters | Storage capacity, ET extraction depth, drying and recharge timing | SMAP soil moisture time series | ET and recharge partitioning | High-value calibration parameter family. |
| LAI by PFT | Notebook lookup values | PET and vegetation water use | SMAP soil moisture seasonal / event trajectory | Land-cover consistency | Secondary priority unless independent LAI data are available. |
| Vegetation cover fraction rule (LAI/4) | Notebook rule | Interception and ET-related behavior | SMAP soil moisture trajectory | Physical plausibility | Secondary heuristic parameter. |
| T_snow and T_rain thresholds | Snow partition block | Rain-vs-snow split | ERA5 SWE time series / snow timing | SMAP timing indirectly | High priority for winter storms. |
| melt_factor and T_base | Degree-day melt block | Snowmelt timing and water input | ERA5 SWE depletion curve | Water input timing and recharge pulse | High priority where snow controls the storm response. |
| initial_swe_mm | Snow initialization | Beginning snow storage | ERA5 SWE at event start | Water-input timing | Should be matched to external SWE estimate before tuning melt factor. |
| Recharge scenario choice: local max_recharge vs routed_recharge_max | Notebook scenario branch | Spatial structure of landslide forcing | Observed landslide locations / runout-initiation zones | Wetness realism in convergent terrain | Critical structural choice; compare both explicitly. |
| Recharge uncertainty factor (std = 0.1 * mean) | LandslideProbability setup | Monte Carlo spread of recharge | Observed uncertainty / sensitivity behavior | Probability spread and map smoothness | Sensitivity parameter, not directly observable. |
| Cohesion reduction by burn severity class | post_fire_cohesion scenario logic | Shear strength reduction after fire | Mapped post-fire landslide initiation locations | Burn severity vs failure relationship | Very important for post-fire calibration. Currently inactive in active notebook cell. |
| Cohesion rasters min/mode/max | Imported static layers | Strength distribution | Observed landslide locations and known stable areas | Physical plausibility by lithology / soils | Calibrate as bounded scenario surfaces, not arbitrary free fields. |
| Internal friction angle raster | Imported static layer | Strength term tan(phi) | Observed landslide locations | Soil texture consistency | Usually lower-priority than recharge and cohesion unless source quality is poor. |
| Boundary-condition elevation cutoff (337 m) and routing setup | DEM / routing setup | Drainage network and contributing area | Known outlet / watershed behavior | Recharge routing realism | Validate structurally before calibrating hydrology. |
| number_of_iterations | LandslideProbability | Monte Carlo sampling noise only | None | Numerical stability of Pf estimate | Not a calibration parameter; use for convergence testing only. |

# 5. Recommended Calibration Targets

| Observable target | What it constrains best | How to compare | Priority |
| --- | --- | --- | --- |
| SMAP daily soil moisture | Initial saturation, ksat_factor, root-zone depth, field/wilting thresholds | AOI/core-node mean volumetric soil moisture time series | High |
| SMAP sub-daily Pacific-time timestamps | Timing mismatch and aggregation issues | Overlay hourly/sub-daily SMAP with daily model states for timing diagnosis | Medium |
| ERA5 SWE | Snow thresholds, melt_factor, T_base, initial SWE | Daily SWE mean and depletion timing | High for winter events |
| Mapped landslide initiation locations / polygons | Recharge scenario choice, cohesion reduction, strength fields, transmissivity assumptions | Presence/absence, hit rate, precision-recall, spatial overlap, rank statistics | Highest for final hazard calibration |
| Known non-failure/stable areas | False-positive control in landslide probability | Specificity / precision diagnostics | High |
| Runoff or streamflow if available later | Hydrologic partitioning and recharge realism | Daily event hydrograph or basin runoff index | High when available |

# 6. Practical Calibration Sequence

1. Validate static data contract first: alignment, CRS, nodata, units, and required field names.

2. Match event-start soil moisture using SMAP-based initialization.

3. Calibrate winter hydrology timing using SWE targets: T_snow, T_rain, melt_factor, T_base, initial SWE.

4. Calibrate soil-water partitioning using SMAP: ksat_factor, root-zone depths, field/wilting thresholds.

5. Compare local recharge vs routed recharge as alternative landslide forcings.

6. Calibrate post-fire strength reduction and cohesion scenarios using observed landslide initiation locations.

7. Only after the above, evaluate the final landslide probability map as the integrated hazard product.

# 7. Bottom Line

For this workflow, the most defensible split is: raw/observed layers stay fixed as prepared inputs; derived variables are deterministic transformations that should be traceable; and calibration should focus on a smaller set of uncertain hydrologic and strength parameters. In the current notebook, the highest-value calibration levers are the initial water state, conductivity / transmissivity scaling, snow partition and melt parameters, root-zone depth family, recharge scenario choice, and post-fire cohesion reduction. Final hazard calibration should be done against mapped landslide locations, while intermediate hydrology should be calibrated against soil moisture and SWE observations.